DAQ channel config (auto-load, 1x per computer)

| portNum | channelName | ioType | signalType | TerminalConfig | Range | ProtType | ProtFunc | ProtID |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ai1 | ThermodeA | input | Voltage | SingleEnded | [-5 5] | thermode | rec | A |
| ai2 | ThermodeB | input | Voltage | SingleEnded | [-5 5] | thermode | rec | B |
| ai3 | Aurora Force OUT | input | Voltage | SingleEnded |  | aurora | force |  |
| ai4 | Aurora Length OUT | input | Voltage | SingleEnded |  | aurora | length |  |
| ai5 | DC Temperature Controller | input | Voltage | SingleEnded |  | heatpad | rec |  |
| port0/line1 | TTL Thermode Onset | input | Digital |  |  | thermode | onset |  |
| port0/line20 | TTL ScanImage Res Galvo | input | Digital |  |  | galvo | res |  |
| port0/line21 | TTL ScanImage Y Galvo | input | Digital |  |  | galvo | y |  |
| port0/line3 | TTL ScanImage: Acq Start | output | Digital |  |  | galvo | scanimage |  |
| port0/line4 | TTL Basler: Frame Trigger | output | Digital |  |  | camera | trigger | A |
| port0/line23 | TTL Basler: UNUSED | output | Digital |  |  | camera | unused | A |
| port0/line0 | BLUE LED | output | Digital |  |  | led | stim | Blue |
| port0/line25 | GREEN LED | output | Digital |  |  | led | stim | Green |
| port0/line26 | Speaker | output | Digital |  |  | audio |  |  |
| port0/line27 | Optical Stimulus | output | Digital |  |  | optical |  |  |
| port0/line28 | HamamatsuTrigger | output | Digital |  |  | camera | trigger | B |
| port0/line29 | LED | output | Digital |  |  | led | flash | white |
| port0/line30 | IRLED_BaslerImaging | output | Digital |  |  | led | flash | IR |
| port0/line2 | Vibration | output | Digital |  |  | actuator | vibration |  |
| port0/line7 | Thermode Trigger | output | Digital |  |  | thermode | trigger |  |
| ao0 | Aurora Force Command Voltage | output | Voltage | SingleEnded | [0 10] | aurora | command |  |

Hardware configs (auto load, 1x per computer)

- Json files, includes mapped protocol names!

Enabled Hardware / Map (based on computer and named setup, selected 1x per session with a default that doesn't need selecting)

Protocol Files

I think I'm just going to code a legacy adaptor rather than trying to integrate it into this system.

NB default behaviour for tPre / tPost?

- Maybe by default throw an error if tPost is shorter than stim length

  - Some override behaviour

General (first line)

nProtRunsX (previously nProtRep): number of times the protocol runs (including the first) - defaults to 1

RandomiseX: how to randomise trials

dPauseX: time between trials in the protocol

(optional)

nTrialRunsX: change the default number of times each stimulus line is run in the trial

~

Trial Info

- Recording paradigms live in hardware configuration, unless you want some stimulus (e.g. LED stimulus) to line up precisely with camera

- By default, all stimuli happen simultaneously, with the same start time

- Each line in this block is a trial.

- For each trial, you can define nTrialRunsX - the number of times the trial is repeated during each run of the protocol.

Stimulus Naming Conventions

In the simplest case, the name of the stimulus is what maps it to attached hardware. To target, e.g. the blue LED as defined in either DaqChanParams (see left) or the computer's HardwareMap (TBD)

In each block of stimuli, the following keywords can be defined exactly once:

- repDelXXXX - delay between repeats in ms. Default 0.

- startDelXXXX - start delay from stimulus t=0

- nStimsX (previously nRepX) - number of times the stimulus / stim block is run within a trial. Default 1.

-1 will make the stimulus repeat until the end of tPost.

StimA StimB nStims2 repDel1000

| StimA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |

Note the repeat delay happens after the conclusion of the longest stim.

To build more complex patterns of stimuli, use bracket notation. Functions in brackets are built independently from the middle out.

(StimA nStims3 repDel1000)(StimB StimC nStims2 repDel1000)

| StimA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |
| StimC |  |  |  |  |  |

Note that because B and C are within the same bracket block, they have the same t=0 for repeats

To build a sequence of stimuli, use the > operator.

(StimA nStims2 repDel1000) > B

| StimA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |

(StimA nStims2 repDel1000) > (B startDel1000)

| StimA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |

Note the startDel operator

More Complex Combinations: Oddballs

To construct an oddball trial, use the oddX notation, which takes the following params in square brackets:

Default stimulus

Stimulus to swap in (can be a single stimulus block, or multiple stimuli separated by | or > )

Odd1[StimA, StimB] nStims4

| StimA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |

Defaults to StimA, and swaps in StimB after every 1 occurrence of StimA

Odd1[StimA, StimB] StimC nStims4 repDel1000

| StimA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |  |  |
| StimC |  |  |  |  |  |  |  |  |  |

Note that when a non-oddball stim is added to the same stimulus block, t=0 remains synchronised

Odd2[StimA, StimB] nStims4 repDel1000

| StimA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |  |

Swap out A after every 2 occurrences, for a total of 4 stimuli

You can swap in a random stimulus out of a selection using the | operator

Odd1[StimA, (StimB | StimC)] nStims4 repDel1000

| StimA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |
| StimC |  |  |  |  |  |  |  |

Or

| StimA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |
| StimC |  |  |  |  |  |  |  |

**NB do we want enforced fairness?**

You can also swap in oddballs in a specific order using the > operator

Odd1[StimA, StimB>StimC)] nStims4 repDel1000

| StimA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |
| StimC |  |  |  |  |  |  |  |

Odd1[StimA, (StimC>StimB)] nStims4 repDel1000

| StimA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |
| StimC |  |  |  |  |  |  |  |

Odd0[, (StimC>StimB)] nStims4 repDel1000

| StimA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| StimB |  |  |  |  |  |  |  |
| StimC |  |  |  |  |  |  |  |

Odd0 lets you randomise / sequence through all the stims in the second arg

~

Stimulus Info

- Specific stimuli are defined here. Depending on the stimulus type, as defined by the prefix of the stimulus, different parameters will be available for use. Definitions shown below. Note that capitalisation consistency is vital for stimulus subtypes (e.g. CameraA, not Cameraa or cameraa)

- Stimuli are then sub-defined by their number. For example, if you want VibratorA to function differently in trials 1 and 2, you would define VibratorA1 and VibratorA2. To target all vibration motors, just say Vibration1

- Functions can be given multiple names (e.g. if you want a vibration output to be sent to one motor in trial 1 and a different motor in trial 2, you can define it as follows)

- NB ALLOW LINKING TO OTHER STIMS - e.g. only trigger camera between stim LEDs?

Define a stimulus function using the following syntax:

StimulusName(type): param1 param2 param3

You can set multiple hardware devices to be connected to the same stimulus using the & operator

StimulusName1 & StimulusName2(type): param1 param2 param3

Parameters are defined using their name and then a number directly after - NO SPACES. When defaults are given, defining the parameter is optional.

Supported types and parameters:

Thermode

| Nxxx | set neutral temperature in 1/10 °C (xxx = 200-400) |
| --- | --- |
| Sxxxxx | enable/disable area 1 to 5 (x=0: disable, x=1: enable |
| Cxxx | set stimulation temperature in 1/10 °C (s=0: all areas or s=1-5, xxx=100-600) |
| Vxxxx | set stimulation speed in 1/10 °C/s (s=0: all areas or s=1-5, xxxx=0001-9999) |
| Dxxxx | set stimulation duration in ms (s=0: all areas or s=1-5, xxxx=0001-9999) |
| Txxx | set trigger number (xxx = 001-255) |
| Ix | enable/disable Integral term (x=0: disable, x=1: enable) |

pwm

| DC | Duty cycle (%) |
| --- | --- |
| Freq | Pulse frequency (Hz) |
| Dur | PWM duration (ms) |
| upRampDur | Duration of any up ramp (ms). Takes place within Dur. Default 0 |
| downRampDur | Duration of any down ramp (ms) Takes place within Dur. Default 0 |

sine

| Amp | Peak amplitude (V) |
| --- | --- |
| Freq | Sinewave frequency (Hz) |
| Dur | Duration (ms) |
| upRampDur | Duration of any up ramping for amplitude (ms). Takes place within Dur. Default 0 |
| downRampDur | Duration of any down ramping for amplitude (ms). Takes place within Dur. Default 0 |

analog

| Amp | Amplitude (V) |
| --- | --- |
| Dur | Pulse duration (ms) |
| upRampDur | Duration of any up ramping for amplitude (ms). Takes place within Dur. Default 0 |
| downRampDur | Duration of any down ramping for amplitude (ms). Takes place within Dur. Default 0 |

Arbitrary

full_path_to_txt_file.csv

dig

| Dur | Digital onset duration (ms) |
| --- | --- |

repeatTrig

| PW | Pulse width (ms) |
| --- | --- |
| Freq | Pulse frequency (Hz) |
| Dur | Trigger duration (ms) (exclusive with n - do not define both!) |
| n | Number of triggers (exclusive with Dur - do not define both!) |

Connected(? - could be linked to a stim with a startDel)

~